#include <Arduino.h>

#include <Wire.h>

#include <WiFiS3.h>

#include <PubSubClient.h>

// ===== Wi-Fi & MQTT =====

const char* SSID         = "KDT_Team3";

const char* PASSWORD     = "1q2w3e4r";

const char* MQTT_SERVER  = "192.168.0.6";

const uint16_t MQTT_PORT = 1883;

const char* TOPIC_SPEED_A   = "motor/speed";

const char* TOPIC_SPEED_B   = "motor/speed2";

const char* TOPIC_WARN      = "control/warning";

// 로드셀 → FSR (가속/브레이크) [토픽 이름은 그대로 유지]

const char* TOPIC_LOAD_ACCEL   = "sensor/loadcell";        // 원시 ADC(0~4095)

const char* TOPIC_LOAD_BRAKE   = "sensor/brake_loadcell";  // 원시 ADC(0~4095)

const char* TOPIC_ACCEL_PRESSED = "sensor/accel_pressed";  // 0.0~1.0

const char* TOPIC_BRAKE_PRESSED = "sensor/brake_pressed";  // 0.0~1.0

// IMU 가속도(g): 페달 X(상하), Z(앞뒤)

const char* TOPIC_AX_ACCEL    = "sensor/accel_pedal_x";

const char* TOPIC_AZ_ACCEL    = "sensor/accel_pedal_z";

const char* TOPIC_AX_BRAKE    = "sensor/brake_accel_x";

const char* TOPIC_AZ_BRAKE    = "sensor/brake_accel_z";

WiFiClient   wifiClient;

PubSubClient mqtt(wifiClient);

// ===== 모터 핀/엔코더 =====

const int RPWM_A = 5;

const int LPWM_A = 6;

const int ENC_A1 = 2;

const int RPWM_B = 10;

const int LPWM_B = 9;

const int ENC_A2 = 3;

volatile long pulsesA = 0;

volatile long pulsesB = 0;

const float PPR       = 500.0f;

float speedA          = 0.0f;

float speedB          = 0.0f;

unsigned long lastT   = 0;

int pwmA              = 255;

int pwmB              = 255;

const int WARN_PWM    = 0;   // 경고 시 즉시 급정지(=0)

// ===== MPU6500 =====

#define REG_PWR_MGMT_1   0x6B

#define REG_ACCEL_XOUT_H 0x3B

#define MPU_ADDR_ACCEL   0x68

#define MPU_ADDR_BRAKE   0x69

// ====== FSR (아날로그) ======

const int FSR_ACCEL_PIN = A1;     // 가속 페달 FSR 노드

const int FSR_BRAKE_PIN = A3;     // 브레이크 FSR 노드

const float R_FIXED_OHM = 10000.0f;   // 전압 분배기 기준저항 10kΩ

const int   ADC_BITS    = 12;         // UNO R4: 12비트

const float ADC_MAX     = (1 << ADC_BITS) - 1;  // 4095

// 간단 캘리브레이션(원하는 값으로 바꿔서 사용)

// - ZERO: 발 안 올린 상태에서의 평균 ADC

// - CAL : 일정 힘(예: 20kgf)에 해당하는 ADC

// - CAL_WEIGHT: 위 CAL에 해당하는 "의미있는 크기"(여기선 1.0로 정규화)

float FSR_A_ZERO = 200.0f;     // 예시값: 실제 측정으로 갱신 권장

float FSR_A_CAL  = 2000.0f;    // 예시값: 실제 측정으로 갱신 권장

float FSR_B_ZERO = 200.0f;     // 예시값

float FSR_B_CAL  = 2000.0f;    // 예시값

const float CAL_WEIGHT = 1.0f; // 1.0 → 0.0~1.0 정규화

// --- ISR ---

void onPulseA() { pulsesA++; }

void onPulseB() { pulsesB++; }

// --- MPU6500 유틸 ---

bool mpuWrite(uint8_t addr, uint8_t reg, uint8_t data) {

Wire.beginTransmission(addr);

Wire.write(reg);

Wire.write(data);

return (Wire.endTransmission() == 0);

}

bool mpuReadN(uint8_t addr, uint8_t reg, uint8_t n, uint8_t* buf) {

Wire.beginTransmission(addr);

Wire.write(reg);

if (Wire.endTransmission(false) != 0) return false;

if (Wire.requestFrom(addr, n, true) != n) return false;

for (uint8_t i = 0; i < n; i++) buf[i] = Wire.read();

return true;

}

bool initMPU(uint8_t addr) {

if (!mpuWrite(addr, REG_PWR_MGMT_1, 0x00)) return false;

delay(10);

return true;

}

bool readAccelRaw(uint8_t addr, int16_t& ax, int16_t& ay, int16_t& az) {

uint8_t buf[6];

if (!mpuReadN(addr, REG_ACCEL_XOUT_H, 6, buf)) return false;

ax = (int16_t)((buf[0] << 8) | buf[1]);

ay = (int16_t)((buf[2] << 8) | buf[3]);

az = (int16_t)((buf[4] << 8) | buf[5]);

return true;

}

// --- Wi-Fi ---

void ensureWiFi() {

if (WiFi.status() == WL_CONNECTED) return;

WiFi.disconnect();

WiFi.begin(SSID, PASSWORD);

Serial.print("Wi-Fi 연결 중");

while (WiFi.status() != WL_CONNECTED) { delay(300); Serial.print("."); }

Serial.println("\nWi-Fi 연결됨");

}

// --- MQTT 콜백 ---

void mqttCallback(char* topic, byte* payload, unsigned int len) {

String msg; msg.reserve(len);

for (unsigned int i = 0; i < len; i++) msg += (char)payload[i];

msg.trim();

if (String(topic) == TOPIC_WARN) {

String s = msg; s.toLowerCase();

if (s == "warning" || s == "1" || s == "on" || s == "true") {

pwmA = WARN_PWM; pwmB = WARN_PWM;

Serial.println("경고 수신 → 즉시 급정지(PWM=0)");

}

}

}

void ensureMQTT() {

if (mqtt.connected()) return;

mqtt.setServer(MQTT_SERVER, MQTT_PORT);

mqtt.setCallback(mqttCallback);

Serial.print("MQTT 연결 중");

while (!mqtt.connected()) {

String cid = "UNO-R4-"; cid += String((uint32_t)millis(), HEX);

if (mqtt.connect(cid.c_str())) {

Serial.println("\nMQTT 연결됨");

mqtt.subscribe(TOPIC_WARN);

} else { Serial.print("."); delay(600); }

}

}

// --- MQTT 발행 유틸 ---

void publishFloatWithPrefix(const char* topic, float val, const char* prefix) {

char buf[32];

dtostrf(val, 0, 2, buf);

String msg(prefix);

msg += buf;

mqtt.publish(topic, msg.c_str());

}

void publishSpeed(const char* topic, float val) { publishFloatWithPrefix(topic, val, "speed:"); }

void publishNumber(const char* topic, float val, int digits = 3) {

char buf[24];

dtostrf(val, 0, digits, buf);

mqtt.publish(topic, buf);

}

// ===== LPF =====

struct LPF1 {

float alpha;

float y;

LPF1(): alpha(0.2f), y(0.0f) {}

float step(float x) { y += alpha * (x - y); return y; }

};

LPF1 lpf_ax_accel, lpf_az_accel;

LPF1 lpf_ax_brake, lpf_az_brake;

// ===== 유틸: FSR 정규화 =====

// ZERO, CAL을 기반으로 0.0~1.0로 맵핑 (하한 0, 상한 1)

float fsrNormalize(float raw, float ZERO, float CAL, float calWeight) {

float num = (raw - ZERO);

float den = (CAL - ZERO);

if (den <= 1e-6f) return 0.0f;

float x = num / den;

if (x < 0) x = 0;

if (x > 1) x = 1;

return x * calWeight; // 기본은 0~1.0

}

// ===== SETUP =====

void setup() {

Serial.begin(115200);

delay(300);

Serial.println("setup 시작");

// ADC 해상도 고정

analogReadResolution(ADC_BITS);

pinMode(RPWM_A, OUTPUT);

pinMode(LPWM_A, OUTPUT);

pinMode(RPWM_B, OUTPUT);

pinMode(LPWM_B, OUTPUT);

pinMode(ENC_A1, INPUT_PULLUP);

pinMode(ENC_A2, INPUT_PULLUP);

attachInterrupt(digitalPinToInterrupt(ENC_A1), onPulseA, RISING);

attachInterrupt(digitalPinToInterrupt(ENC_A2), onPulseB, RISING);

Wire.begin();

Wire.setClock(100000);

initMPU(MPU_ADDR_ACCEL);

initMPU(MPU_ADDR_BRAKE);

ensureWiFi();

ensureMQTT();

lastT = millis();

Serial.println("setup 완료");

}

// ===== LOOP =====

void loop() {

ensureWiFi();

ensureMQTT();

mqtt.loop();

analogWrite(RPWM_A, pwmA);

digitalWrite(LPWM_A, LOW);

analogWrite(RPWM_B, pwmB);

digitalWrite(LPWM_B, LOW);

unsigned long now = millis();

if (now - lastT >= 100) {

float dt = (now - lastT) / 1000.0f;

lastT = now;

noInterrupts();

long cntA = pulsesA; pulsesA = 0;

long cntB = pulsesB; pulsesB = 0;

interrupts();

speedA = (cntA / PPR) * 2.0f * PI / dt;

speedB = (cntB / PPR) * 2.0f * PI / dt;

// --- IMU 읽기 ---

int16_t ax_a=0, ay_a=0, az_a=0;

readAccelRaw(MPU_ADDR_ACCEL, ax_a, ay_a, az_a);

float ax_accel_g = (float)ax_a / 16384.0f;

float az_accel_g = (float)az_a / 16384.0f;

float ax_accel_lpf = lpf_ax_accel.step(ax_accel_g);

float az_accel_lpf = lpf_az_accel.step(az_accel_g);

int16_t ax_b=0, ay_b=0, az_b=0;

readAccelRaw(MPU_ADDR_BRAKE, ax_b, ay_b, az_b);

float ax_brake_g = (float)ax_b / 16384.0f;

float az_brake_g = (float)az_b / 16384.0f;

float ax_brake_lpf = lpf_ax_brake.step(ax_brake_g);

float az_brake_lpf = lpf_az_brake.step(az_brake_g);

// --- FSR 읽기 (원시 ADC) ---

int rawA = analogRead(FSR_ACCEL_PIN); // 0~4095

int rawB = analogRead(FSR_BRAKE_PIN); // 0~4095

// 정규화 값(0.0~1.0) — 간단 2점 보정

float accel_norm  = fsrNormalize((float)rawA, FSR_A_ZERO, FSR_A_CAL, CAL_WEIGHT);

float brake_norm  = fsrNormalize((float)rawB, FSR_B_ZERO, FSR_B_CAL, CAL_WEIGHT);

// === 시리얼 출력 ===

Serial.print("[A] "); Serial.print(speedA, 3);

Serial.print(" rad/s | [B] "); Serial.print(speedB, 3);

Serial.print(" rad/s");

Serial.print(" | accel_AX: "); Serial.print(ax_accel_lpf, 4);

Serial.print(" g, AZ: ");      Serial.print(az_accel_lpf, 4);

Serial.print(" | brake_AX: "); Serial.print(ax_brake_lpf, 4);

Serial.print(" g, AZ: ");      Serial.print(az_brake_lpf, 4);

Serial.print(" | FSR_brake(raw): "); Serial.print(rawA);

Serial.print(" |  FSR_accel(raw): "); Serial.print(rawB);

Serial.print(" | accel_norm: "); Serial.print(accel_norm, 3);

Serial.print(" | brake_norm: "); Serial.print(brake_norm, 3);

Serial.println();

// === MQTT 발행 ===

publishSpeed(TOPIC_SPEED_A, speedA);

publishSpeed(TOPIC_SPEED_B, speedB);

publishNumber(TOPIC_AX_ACCEL, ax_accel_lpf, 4);

publishNumber(TOPIC_AZ_ACCEL, az_accel_lpf, 4);

publishNumber(TOPIC_AX_BRAKE, ax_brake_lpf, 4);

publishNumber(TOPIC_AZ_BRAKE, az_brake_lpf, 4);

// 원시 ADC 값 발행(기존 로드셀 토픽 재사용)

publishNumber(TOPIC_LOAD_ACCEL, (float)rawA, 0);

publishNumber(TOPIC_LOAD_BRAKE, (float)rawB, 0);

// “pressed 값”은 0.0~1.0 정규화본으로 발행

publishNumber(TOPIC_ACCEL_PRESSED, accel_norm, 3);

publishNumber(TOPIC_BRAKE_PRESSED, brake_norm, 3);

}

}