[

{

"id": "5124c2cc8eafaa38",

"type": "tab",

"label": "Speed Monitoring with DB",

"disabled": false,

"info": ""

},

{

"id": "afa494f11d498e1f",

"type": "ui_gauge",

"z": "5124c2cc8eafaa38",

"name": "Speed Gauge",

"group": "08f0486f70b9dc3f",

"order": 0,

"width": "0",

"height": "0",

"gtype": "gage",

"title": "Speed (km/h)",

"label": "km/h",

"format": "{{value}}",

"min": 0,

"max": "100",

"colors": [

"#00b500",

"#e6e600",

"#ca3838"

],

"seg1": "",

"seg2": "",

"diff": false,

"className": "",

"x": 620,

"y": 80,

"wires": []

},

{

"id": "336ca27b7861ebb6",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "speed gauge, chart",

"func": "let p = Buffer.isBuffer(msg.payload) ? msg.payload.toString('utf8') : String(msg.payload || '');\np = p.trim();\nlet m = p.match(/-?\\d+(?:\\.\\d+)?/);\nif (!m) { return null; }              // 실패면 표시하지 않음 (이전 값 유지)\nmsg.payload = Number(m[0]);           // 성공 시만 표시\nreturn msg;\n",

"outputs": 1,

"timeout": 0,

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 390,

"y": 80,

"wires": [

[

"afa494f11d498e1f"

]

]

},

{

"id": "af9e1e369d52c1b4",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "",

"topic": "motor/speed",

"qos": "0",

"datatype": "utf8",

"broker": "af94e945c33ea222",

"nl": false,

"rap": true,

"rh": 0,

"inputs": 0,

"x": 190,

"y": 80,

"wires": [

[

"336ca27b7861ebb6"

]

]

},

{

"id": "7d2c254fede041c3",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "brake/foot",

"topic": "brake/foot",

"qos": "2",

"datatype": "auto-detect",

"broker": "af94e945c33ea222",

"nl": false,

"rap": false,

"rh": 0,

"inputs": 0,

"x": 170,

"y": 260,

"wires": [

[

"75408e2eb79cc803",

"58bd6de13e07c03a"

]

]

},

{

"id": "487847b4215be1ac",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "sensor/brake_loadcell",

"topic": "sensor/brake_loadcell",

"qos": "2",

"datatype": "auto-detect",

"broker": "af94e945c33ea222",

"nl": false,

"rap": false,

"rh": 0,

"inputs": 0,

"x": 210,

"y": 320,

"wires": [

[

"7393ccd0fb7145f5"

]

]

},

{

"id": "60c1e9f714bc718f",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "sensor/brake_accel_x",

"topic": "sensor/brake_accel_x",

"qos": "2",

"datatype": "auto-detect",

"broker": "af94e945c33ea222",

"nl": false,

"rap": false,

"rh": 0,

"inputs": 0,

"x": 210,

"y": 380,

"wires": [

[

"21ca28de08f20633"

]

]

},

{

"id": "7393ccd0fb7145f5",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "foot,load,accel,accel_z",

"func": "// 세 토픽(brake/foot, sensor/loadcell, sensor/accel)의 최신값을 context에 저장하고,\n// 모두 유효할 때 조건(foot===1 && load>=20 && accel===1) 충족 시 warning 발행.\n\nlet state = context.get('state') || { foot: undefined, load: undefined, accel: undefined , accel_z: undefined};\n\nconst topic = msg.topic;\nlet val = msg.payload;\n\n// payload 정규화 (Buffer/문자열 → 문자열 → 숫자 등)\nif (Buffer.isBuffer(val)) val = val.toString();\nif (typeof val === 'string') val = val.trim();\n\nif (topic === 'brake/foot') {\n    state.foot = Number(val);\n}\nelse if (topic === 'sensor/brake_loadcell') {\n    state.load = Number(val);\n}\nelse if (topic === 'sensor/brake_accel_x' || topic === 'accel') {\n    // 아두이노에서 가속도를 숫자 문자열로 보냄 → 숫자화\n    state.accel = Number(val);\n}\nelse if (topic === 'sensor/brake_accel_z' || topic === 'accel_z') {\n    // 아두이노에서 가속도를 숫자 문자열로 보냄 → 숫자화\n    state.accel_z = Number(val);\n}\nelse {\n    context.set('state', state);\n    return null; // 관심 없는 토픽\n}\n\ncontext.set('state', state);\n\n// 상태 표시(디버그용)\nnode.status({ fill: 'blue', shape: 'dot', text: `foot=${state.foot} load=${state.load} accel=${state.accel} accel_z=${state.accel_z}` });\n\n// 모든 값이 유효할 때만 판정\nconst hasAll = (state.foot !== undefined) && (state.load !== undefined) && (state.accel !== undefined) && (state.accel_z !== undefined);\n\nif (hasAll) {\n    if (state.foot === 1 && !Number.isNaN(state.load) && state.load >= 20 && state.accel === 1 && state.accel_z === 1) {\n        return { topic: 'control/warning', payload: 'warning' };\n    } else {\n        // 필요하면 해제 메시지 발행\n        // return { topic: 'control/warning', payload: 'clear' };\n        return null;\n    }\n}\n\nreturn null;",

"outputs": 1,

"timeout": "",

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 470,

"y": 320,

"wires": [

[

"c4b5395dab0b7c69",

"8f2b08486934bb23",

"130d82df66e0fec2"

]

]

},

{

"id": "c4b5395dab0b7c69",

"type": "mqtt out",

"z": "5124c2cc8eafaa38",

"name": "control/warning",

"topic": "control/warning",

"qos": "",

"retain": "",

"respTopic": "",

"contentType": "",

"userProps": "",

"correl": "",

"expiry": "",

"broker": "af94e945c33ea222",

"x": 750,

"y": 260,

"wires": []

},

{

"id": "8f2b08486934bb23",

"type": "debug",

"z": "5124c2cc8eafaa38",

"name": "발행 확인 (warning/clear)",

"active": true,

"tosidebar": true,

"console": false,

"tostatus": false,

"complete": "true",

"targetType": "full",

"statusVal": "",

"statusType": "auto",

"x": 780,

"y": 320,

"wires": []

},

{

"id": "130d82df66e0fec2",

"type": "ui_text",

"z": "5124c2cc8eafaa38",

"group": "08f0486f70b9dc3f",

"order": 1,

"width": 0,

"height": 0,

"name": "⚠ BRAKE 경고 상태",

"label": "⚠ BRAKE 경고 상태 ",

"format": "{{msg.payload}}",

"layout": "",

"className": "",

"style": false,

"font": "",

"fontSize": "",

"color": "#000000",

"x": 770,

"y": 380,

"wires": []

},

{

"id": "21ca28de08f20633",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "sensor/brake_accel_x",

"func": "// 예: msg.payload = \"accel:5700\"\nconst raw = msg.payload.toString().trim();\n\n// \"accel:\" 부분 제거하고 숫자로 변환\nconst y = Number(raw.replace(/^accel:/i, \"\"));\n\n// 디버그용\nnode.warn(`y=${y}`);\n\n// 조건: y가 0 이하이면 1, 아니면 0\nmsg.topic = \"accel\";\nmsg.payload = (y >= -0.88) ? 1 : 0;\n\nreturn msg;",

"outputs": 1,

"timeout": 0,

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 470,

"y": 380,

"wires": [

[

"7393ccd0fb7145f5"

]

]

},

{

"id": "dbde57746b297ebb",

"type": "ui_text",

"z": "5124c2cc8eafaa38",

"group": "08f0486f70b9dc3f",

"order": 1,

"width": 0,

"height": 0,

"name": "BRAKE 대시보드 경고 표시",

"label": "BRAKE",

"format": "{{msg.payload}}",

"layout": "",

"className": "",

"style": false,

"font": "",

"fontSize": "",

"color": "#000000",

"x": 1070,

"y": 220,

"wires": []

},

{

"id": "57c4135688e9804b",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "motor/speed2",

"func": "// 문자열에서 숫자만 추출: \"speed: 7.42\"\nlet parts = msg.payload.split(\":\");\nif (parts.length < 2) return null;  // \"speed: ???\" 형식이 아니면 무시\n\nlet speed = parseFloat(parts[1].trim());\nif (isNaN(speed)) return null;      // 숫자 추출 실패 시 무시\n\nmsg.topic = `INSERT INTO motor_speed2 (speed) VALUES (${speed})`;\nreturn msg;",

"outputs": 1,

"timeout": "",

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 390,

"y": 1040,

"wires": [

[

"d17e1c86fd277ace"

]

]

},

{

"id": "24e7d28b03e023be",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "",

"topic": "motor/speed2",

"qos": "0",

"datatype": "utf8",

"broker": "af94e945c33ea222",

"nl": false,

"rap": true,

"rh": 0,

"inputs": 0,

"x": 180,

"y": 1040,

"wires": [

[

"57c4135688e9804b"

]

]

},

{

"id": "mqtt_accel_in",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "sensor/accel_x",

"topic": "sensor/accel_pedal_x",

"qos": "0",

"datatype": "auto",

"broker": "af94e945c33ea222",

"nl": false,

"rap": true,

"rh": 0,

"inputs": 0,

"x": 190,

"y": 800,

"wires": [

[

"parse_accel"

]

]

},

{

"id": "parse_accel",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "가속도 파싱(ax,ay,az)",

"func": "// payload 형태: 0.12 / \"0.12\" / \"0.1,0.2,0.3\" / {ay:0.2} / {x:0.1,y:0.2,z:0.3}\nlet ax=null, ay=null, az=null;\nconst p = msg.payload;\nconst num = v => { const n = parseFloat(v); return Number.isFinite(n) ? n : null; };\n\nif (typeof p === 'number') {\n  ay = p;\n} else if (typeof p === 'string') {\n  const s = p.trim();\n  if (/^[+-]?\\d+(\\.\\d+)?$/.test(s)) {\n    ay = num(s);\n  } else {\n    const m = s.match(/(-?\\d+\\.?\\d*)\\s*,\\s*(-?\\d+\\.?\\d*)\\s*,\\s*(-?\\d+\\.?\\d*)/);\n    if (m) { ax=num(m[1]); ay=num(m[2]); az=num(m[3]); }\n  }\n} else if (p && typeof p === 'object') {\n  ax = num(p.ax ?? p.x);\n  ay = num(p.ay ?? p.y);\n  az = num(p.az ?? p.z);\n}\nif (ay === null) return null;\n\n// ★ 플레이스홀더(?, msg.params) 쓰지 말고 완성된 문자열로 보냄\nconst axSQL = (ax===null ? \"NULL\" : ax);\nconst aySQL = (ay===null ? \"NULL\" : ay);\nconst azSQL = (az===null ? \"NULL\" : az);\n\nmsg.topic = `INSERT INTO accel_readings (ax, ay, az, ts) VALUES (${axSQL}, ${aySQL}, ${azSQL}, NOW(6))`;\ndelete msg.params;\nreturn msg;",

"outputs": 1,

"timeout": "",

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 400,

"y": 800,

"wires": [

[

"d17e1c86fd277ace"

]

]

},

{

"id": "mqtt_roadcell_in",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "sensor/brake_loadcell",

"topic": "sensor/brake_loadcell",

"qos": "0",

"datatype": "auto",

"broker": "af94e945c33ea222",

"nl": false,

"rap": true,

"rh": 0,

"inputs": 0,

"x": 210,

"y": 860,

"wires": [

[

"parse_roadcell"

]

]

},

{

"id": "parse_roadcell",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "로드셀 파싱(weight)",

"func": "const p = msg.payload;\nconst num = v => { const n = parseFloat(v); return Number.isFinite(n) ? n : null; };\n\nlet weight = null;\nif (typeof p === 'number')      weight = p;\nelse if (typeof p === 'string') weight = num(p.trim());\nelse if (p && typeof p === 'object') weight = num(p.weight ?? p.value ?? p.w);\nif (weight === null) return null;\n\nmsg.topic = `INSERT INTO roadcell_readings (weight, ts) VALUES (${weight}, NOW(6))`;\ndelete msg.params;\nreturn msg;",

"outputs": 1,

"timeout": "",

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 420,

"y": 860,

"wires": [

[

"d17e1c86fd277ace"

]

]

},

{

"id": "db_ok_debug",

"type": "debug",

"z": "5124c2cc8eafaa38",

"name": "DB Insert OK",

"active": true,

"tosidebar": true,

"console": false,

"tostatus": true,

"complete": "true",

"targetType": "full",

"statusVal": "payload",

"statusType": "auto",

"x": 900,

"y": 840,

"wires": []

},

{

"id": "d17e1c86fd277ace",

"type": "mysql",

"z": "5124c2cc8eafaa38",

"mydb": "mysql_config_id",

"name": "Insert to MariaDB",

"x": 680,

"y": 840,

"wires": [

[

"db_ok_debug"

]

]

},

{

"id": "e1268bfe2fd19221",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "",

"topic": "motor/speed",

"qos": "0",

"datatype": "utf8",

"broker": "af94e945c33ea222",

"nl": false,

"rap": true,

"rh": 0,

"inputs": 0,

"x": 180,

"y": 980,

"wires": [

[

"84f98ab5cd9584d8"

]

]

},

{

"id": "84f98ab5cd9584d8",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "motor/speed",

"func": "// 문자열에서 숫자만 추출: \"speed: 7.42\"\nlet parts = msg.payload.split(\":\");\nif (parts.length < 2) return null;  // \"speed: ???\" 형식이 아니면 무시\n\nlet speed = parseFloat(parts[1].trim());\nif (isNaN(speed)) return null;      // 숫자 추출 실패 시 무시\n\nmsg.topic = `INSERT INTO motor_speed1 (speed) VALUES (${speed})`;\nreturn msg;",

"outputs": 1,

"timeout": "",

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 390,

"y": 980,

"wires": [

[

"d17e1c86fd277ace"

]

]

},

{

"id": "b4022ebd61991ebc",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "sensor/accel_z",

"topic": "sensor/accel_pedal_z",

"qos": "0",

"datatype": "auto",

"broker": "af94e945c33ea222",

"nl": false,

"rap": true,

"rh": 0,

"inputs": 0,

"x": 190,

"y": 740,

"wires": [

[

"0f0f2dc8dfed22f5"

]

]

},

{

"id": "0f0f2dc8dfed22f5",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "가속도 파싱(ax,ay,az)",

"func": "// payload 형태: 0.12 / \"0.12\" / \"0.1,0.2,0.3\" / {ay:0.2} / {x:0.1,y:0.2,z:0.3}\nlet ax=null, ay=null, az=null;\nconst p = msg.payload;\nconst num = v => { const n = parseFloat(v); return Number.isFinite(n) ? n : null; };\n\nif (typeof p === 'number') {\n  ay = p;\n} else if (typeof p === 'string') {\n  const s = p.trim();\n  if (/^[+-]?\\d+(\\.\\d+)?$/.test(s)) {\n    ay = num(s);\n  } else {\n    const m = s.match(/(-?\\d+\\.?\\d*)\\s*,\\s*(-?\\d+\\.?\\d*)\\s*,\\s*(-?\\d+\\.?\\d*)/);\n    if (m) { ax=num(m[1]); ay=num(m[2]); az=num(m[3]); }\n  }\n} else if (p && typeof p === 'object') {\n  ax = num(p.ax ?? p.x);\n  ay = num(p.ay ?? p.y);\n  az = num(p.az ?? p.z);\n}\nif (ay === null) return null;\n\n// ★ 플레이스홀더(?, msg.params) 쓰지 말고 완성된 문자열로 보냄\nconst axSQL = (ax===null ? \"NULL\" : ax);\nconst aySQL = (ay===null ? \"NULL\" : ay);\nconst azSQL = (az===null ? \"NULL\" : az);\n\nmsg.topic = `INSERT INTO accel_readings_a (ax, ay, az, ts) VALUES (${axSQL}, ${aySQL}, ${azSQL}, NOW(6))`;\ndelete msg.params;\nreturn msg;",

"outputs": 1,

"timeout": "",

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 400,

"y": 740,

"wires": [

[

"d17e1c86fd277ace"

]

]

},

{

"id": "dde2ab564f1d61dc",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "MQTT brake/foot",

"topic": "brake/foot",

"qos": "0",

"datatype": "auto-detect",

"broker": "af94e945c33ea222",

"nl": false,

"rap": false,

"rh": 0,

"inputs": 0,

"x": 190,

"y": 1100,

"wires": [

[

"db4346dbc55f2a31"

]

]

},

{

"id": "b54133ed0da60434",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "MQTT accel/foot",

"topic": "accel/foot",

"qos": "0",

"datatype": "auto-detect",

"broker": "af94e945c33ea222",

"nl": false,

"rap": false,

"rh": 0,

"inputs": 0,

"x": 190,

"y": 1160,

"wires": [

[

"77d7c5fd57a5d37d"

]

]

},

{

"id": "db4346dbc55f2a31",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "Parse → cam_brake",

"func": "// ✅ 1) payload를 문자열로 안전 변환\nlet s = Buffer.isBuffer(msg.payload) ? msg.payload.toString('utf8') : String(msg.payload);\ns = s.trim();                 // 앞뒤 공백/개행 제거\n\n// ✅ 2) 0/1 추출 (문자 섞여도 됨: \"brake : 1\", \"1\", \"brake=0\" 등)\nconst m = s.match(/\\b([01])\\b/);\nif (!m) {\n  node.warn(\"invalid brake payload: \" + s);\n  return null;\n}\nconst v = Number(m[1]);       // 0 또는 1\n\n/* ─────────────────────────────────────────────────────────\n   선택 A) MySQL 노드가 '파라미터 사용(msg.params)'을 지원할 때\n   (node-red-node-mysql 최신 버전, 설정에서 'Use prepared statements' 켜기)\n───────────────────────────────────────────────────────── */\n/*msg.topic  = \"INSERT INTO cam_brake (state, ts) VALUES (?, NOW(6))\";\nmsg.params = [v];\nreturn msg;\n*/\n/* ─────────────────────────────────────────────────────────\n   선택 B) 위가 안 되면, 아래 두 줄만 남기고 위 msg.topic/msg.params 는 주석 처리\n   → 숫자만 인라인하므로 SQL 인젝션 위험 없음 (0/1만 허용)\n───────────────────────────────────────────────────────── */\nmsg.topic = `INSERT INTO cam_brake (state, ts) VALUES (${v}, NOW(6))`;\nreturn msg;\n",

"outputs": 1,

"timeout": "",

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 450,

"y": 1100,

"wires": [

[

"892ddc1634322a9a",

"a682de0911d4e4b1"

]

]

},

{

"id": "77d7c5fd57a5d37d",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "Parse → cam_accel",

"func": "let s = Buffer.isBuffer(msg.payload) ? msg.payload.toString('utf8') : String(msg.payload);\ns = s.trim();\nconst m = s.match(/\\b([01])\\b/);\nif (!m) {\n  node.warn(\"invalid accel payload: \" + s);\n  return null;\n}\nconst v = Number(m[1]);\n\n/*// 선택 A) 파라미터 사용\nmsg.topic  = \"INSERT INTO cam_accel (state, ts) VALUES (?, NOW(6))\";\nmsg.params = [v];\nreturn msg;\n*/\n// 선택 B) 값 인라인\nmsg.topic = `INSERT INTO cam_accel (state, ts) VALUES (${v}, NOW(6))`;\nreturn msg;\n",

"outputs": 1,

"timeout": "",

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 450,

"y": 1160,

"wires": [

[

"892ddc1634322a9a",

"a682de0911d4e4b1"

]

]

},

{

"id": "892ddc1634322a9a",

"type": "debug",

"z": "5124c2cc8eafaa38",

"name": "SQL out",

"active": true,

"tosidebar": true,

"console": false,

"tostatus": false,

"complete": "true",

"statusVal": "",

"statusType": "auto",

"x": 730,

"y": 1160,

"wires": []

},

{

"id": "a682de0911d4e4b1",

"type": "mysql",

"z": "5124c2cc8eafaa38",

"mydb": "mysql_config_id",

"name": "Insert to MariaDB",

"x": 760,

"y": 1100,

"wires": [

[]

]

},

{

"id": "75408e2eb79cc803",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "function 1",

"func": "// payload 예시: \"brake : 0\" 또는 \"brake : 1\"\nlet s = msg.payload.toString().trim();  \n\n// \"brake : 0\" 또는 \"brake : 1\" 중 숫자만 추출\nlet m = s.match(/\\b[01]\\b/);  \n\nif (!m) {\n    node.warn(\"invalid brake payload: \" + s);\n    return null;\n}\n\n// 숫자로 변환\nmsg.payload = Number(m[0]);  \n\nreturn msg;\n",

"outputs": 1,

"timeout": 0,

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 390,

"y": 260,

"wires": [

[

"7393ccd0fb7145f5"

]

]

},

{

"id": "0c2692d605cb62d0",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "accel/foot",

"topic": "accel/foot",

"qos": "2",

"datatype": "auto-detect",

"broker": "af94e945c33ea222",

"nl": false,

"rap": false,

"rh": 0,

"inputs": 0,

"x": 170,

"y": 500,

"wires": [

[

"17d54c33c084f54c",

"c3a8d10c5f4836f6"

]

]

},

{

"id": "b59274e3a4e00ae7",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "sensor/loadcell",

"topic": "sensor/loadcell",

"qos": "2",

"datatype": "auto-detect",

"broker": "af94e945c33ea222",

"nl": false,

"rap": false,

"rh": 0,

"inputs": 0,

"x": 190,

"y": 560,

"wires": [

[

"6fb2a386ed65c014"

]

]

},

{

"id": "34203cda1a9f2c19",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "sensor/accel_pedal_x",

"topic": "sensor/accel_pedal_x",

"qos": "2",

"datatype": "auto-detect",

"broker": "af94e945c33ea222",

"nl": false,

"rap": false,

"rh": 0,

"inputs": 0,

"x": 210,

"y": 620,

"wires": [

[

"2925ff568c68fa67"

]

]

},

{

"id": "6fb2a386ed65c014",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "foot,load,accel",

"func": "// 세 토픽(brake/foot, sensor/loadcell, sensor/accel)의 최신값을 context에 저장하고,\n// 모두 유효할 때 조건(foot===1 && load>=20 && accel===1) 충족 시 warning 발행.\n\nlet state = context.get('state') || { foot: undefined, load: undefined, accel: undefined , accel_z: undefined};\n\nconst topic = msg.topic;\nlet val = msg.payload;\n\n// payload 정규화 (Buffer/문자열 → 문자열 → 숫자 등)\nif (Buffer.isBuffer(val)) val = val.toString();\nif (typeof val === 'string') val = val.trim();\n\nif (topic === 'accel/foot') {\n    state.foot = Number(val);\n}\nelse if (topic === 'sensor/loadcell') {\n    state.load = Number(val);\n}\nelse if (topic === 'sensor/accel_pedal_x' || topic === 'accel') {\n    // 아두이노에서 가속도를 숫자 문자열로 보냄 → 숫자화\n    state.accel = Number(val);\n}\nelse if (topic === 'sensor/accel_pedal_z' || topic === 'accel_z') {\n    // 아두이노에서 가속도를 숫자 문자열로 보냄 → 숫자화\n    state.accel_z = Number(val);\n}\nelse {\n    context.set('state', state);\n    return null; // 관심 없는 토픽\n}\n\ncontext.set('state', state);\n\n// 상태 표시(디버그용)\nnode.status({ fill: 'blue', shape: 'dot', text: `foot=${state.foot} load=${state.load} accel=${state.accel} accel_z=${state.accel_z}` });\n\n// 모든 값이 유효할 때만 판정\nconst hasAll = (state.foot !== undefined) && (state.load !== undefined) && (state.accel !== undefined) && (state.accel_z !== undefined);\n\nif (hasAll) {\n    if (state.foot === 0 && !Number.isNaN(state.load) && state.load <= 20 && state.accel === 0 && state.accel_z === 0) {\n        return { topic: 'control/warning', payload: 'warning' };\n    } else {\n        // 필요하면 해제 메시지 발행\n        // return { topic: 'control/warning', payload: 'clear' };\n        return null;\n    }\n}\n\nreturn null;",

"outputs": 1,

"timeout": "",

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 450,

"y": 560,

"wires": [

[

"c140bfb62b194f6b",

"9e811525b20d18c5",

"30495c8802934d14"

]

]

},

{

"id": "c140bfb62b194f6b",

"type": "mqtt out",

"z": "5124c2cc8eafaa38",

"name": "control/warning",

"topic": "control/warning",

"qos": "",

"retain": "",

"respTopic": "",

"contentType": "",

"userProps": "",

"correl": "",

"expiry": "",

"broker": "af94e945c33ea222",

"x": 750,

"y": 500,

"wires": []

},

{

"id": "9e811525b20d18c5",

"type": "debug",

"z": "5124c2cc8eafaa38",

"name": "발행 확인 (warning/clear)",

"active": true,

"tosidebar": true,

"console": false,

"tostatus": false,

"complete": "true",

"targetType": "full",

"statusVal": "",

"statusType": "auto",

"x": 780,

"y": 560,

"wires": []

},

{

"id": "30495c8802934d14",

"type": "ui_text",

"z": "5124c2cc8eafaa38",

"group": "08f0486f70b9dc3f",

"order": 1,

"width": 0,

"height": 0,

"name": "⚠ 경고 상태",

"label": "⚠ ACCEL 경고 상태",

"format": "{{msg.payload}}",

"layout": "",

"className": "",

"style": false,

"font": "",

"fontSize": "",

"color": "#000000",

"x": 740,

"y": 620,

"wires": []

},

{

"id": "2925ff568c68fa67",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "sensor/accel_pedal_x",

"func": "// 예: msg.payload = \"accel:5700\"\nconst raw = msg.payload.toString().trim();\n\n// \"accel:\" 부분 제거하고 숫자로 변환\nconst y = Number(raw.replace(/^accel:/i, \"\"));\n\n// 디버그용\nnode.warn(`y=${y}`);\n\n// 조건: y가 0 이하이면 1, 아니면 0\nmsg.topic = \"accel\";\nmsg.payload = (y >= -0.8) ? 1 : 0;\n\nreturn msg;",

"outputs": 1,

"timeout": 0,

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 470,

"y": 620,

"wires": [

[

"6fb2a386ed65c014"

]

]

},

{

"id": "ac7b8b05731d8de8",

"type": "ui_text",

"z": "5124c2cc8eafaa38",

"group": "08f0486f70b9dc3f",

"order": 1,

"width": 0,

"height": 0,

"name": "ACCEL 대시보드 경고 표시",

"label": "ACCEL",

"format": "{{msg.payload}}",

"layout": "",

"className": "",

"style": false,

"font": "",

"fontSize": "",

"color": "#000000",

"x": 1030,

"y": 460,

"wires": []

},

{

"id": "17d54c33c084f54c",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "function 3",

"func": "// payload 예시: \"brake : 0\" 또는 \"brake : 1\"\nlet s = msg.payload.toString().trim();  \n\n// \"brake : 0\" 또는 \"brake : 1\" 중 숫자만 추출\nlet m = s.match(/\\b[01]\\b/);  \n\nif (!m) {\n    node.warn(\"invalid accel payload: \" + s);\n    return null;\n}\n\n// 숫자로 변환\nmsg.payload = Number(m[0]);  \n\nreturn msg;\n",

"outputs": 1,

"timeout": 0,

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 390,

"y": 500,

"wires": [

[

"6fb2a386ed65c014"

]

]

},

{

"id": "51d4253f2c4ab0f5",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "sensor/accel_pedal_z",

"topic": "sensor/accel_pedal_z",

"qos": "0",

"datatype": "auto",

"broker": "af94e945c33ea222",

"nl": false,

"rap": true,

"rh": 0,

"inputs": 0,

"x": 210,

"y": 680,

"wires": [

[

"35adcd29c13e2fc2"

]

]

},

{

"id": "35adcd29c13e2fc2",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "sensor/accel_pedal_z",

"func": "// 예: msg.payload = \"accel:5700\"\nconst raw = msg.payload.toString().trim();\n\n// \"accel:\" 부분 제거하고 숫자로 변환\nconst y = Number(raw.replace(/^accel:/i, \"\"));\n\n// 디버그용\nnode.warn(`y=${y}`);\n\n// 조건: y가 0 이하이면 1, 아니면 0\nmsg.topic = \"accel_z\";\nmsg.payload = (y <= -0.7) ? 1 : 0;\n\nreturn msg;",

"outputs": 1,

"timeout": 0,

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 470,

"y": 680,

"wires": [

[

"6fb2a386ed65c014"

]

]

},

{

"id": "dc743c5fa40154b5",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "sensor/brake_accel_z",

"topic": "sensor/brake_accel_z",

"qos": "0",

"datatype": "auto",

"broker": "af94e945c33ea222",

"nl": false,

"rap": true,

"rh": 0,

"inputs": 0,

"x": 210,

"y": 440,

"wires": [

[

"c0de5c9e8ea8cc68"

]

]

},

{

"id": "c0de5c9e8ea8cc68",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "sensor/brake_accel_z",

"func": "// 예: msg.payload = \"accel:5700\"\nconst raw = msg.payload.toString().trim();\n\n// \"accel:\" 부분 제거하고 숫자로 변환\nconst y = Number(raw.replace(/^accel:/i, \"\"));\n\n// 디버그용\nnode.warn(`y=${y}`);\n\n// 조건: y가 0 이하이면 1, 아니면 0\nmsg.topic = \"accel_z\";\nmsg.payload = (y >= 0.5) ? 1 : 0;\n\nreturn msg;",

"outputs": 1,

"timeout": 0,

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 470,

"y": 440,

"wires": [

[

"7393ccd0fb7145f5"

]

]

},

{

"id": "bee58a91ef6e5ffa",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "sensor/loadcell",

"topic": "sensor/loadcell",

"qos": "0",

"datatype": "auto",

"broker": "af94e945c33ea222",

"nl": false,

"rap": true,

"rh": 0,

"inputs": 0,

"x": 190,

"y": 920,

"wires": [

[

"20d7efa87ba10894"

]

]

},

{

"id": "20d7efa87ba10894",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "로드셀 파싱(weight)",

"func": "const p = msg.payload;\nconst num = v => { const n = parseFloat(v); return Number.isFinite(n) ? n : null; };\n\nlet weight = null;\nif (typeof p === 'number')      weight = p;\nelse if (typeof p === 'string') weight = num(p.trim());\nelse if (p && typeof p === 'object') weight = num(p.weight ?? p.value ?? p.w);\nif (weight === null) return null;\n\nmsg.topic = `INSERT INTO roadcell_readings_a (weight, ts) VALUES (${weight}, NOW(6))`;\ndelete msg.params;\nreturn msg;",

"outputs": 1,

"timeout": "",

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 400,

"y": 920,

"wires": [

[

"d17e1c86fd277ace"

]

]

},

{

"id": "1e9e4d59434d4425",

"type": "http in",

"z": "5124c2cc8eafaa38",

"name": "GET /api/:table",

"url": "/api/:table",

"method": "get",

"upload": false,

"swaggerDoc": "",

"x": 240,

"y": 1360,

"wires": [

[

"c3ec7ab769cef0e1"

]

]

},

{

"id": "c3ec7ab769cef0e1",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "Build SQL (raw + speedlogs 별칭)",

"func": "// Path param :table, Query: ?limit=200&offset=0\nconst p = msg.req.params || {};\nconst q = msg.req.query || {};\n\n// 허용 테이블/별칭 (식별자 인젝션 방지)\nconst ALLOW = {\n  accel_readings: { order: 'ts' },\n  roadcell_readings: { order: 'ts' },\n  accel_readings_a: { order: 'ts' },\n  roadcell_readings_a: { order: 'ts' },\n  motor_speed1: { order: 'created_at' },\n  motor_speed2: { order: 'created_at' },\n  cam_brake: { order: 'ts' },\n  cam_accel: { order: 'ts' },\n  // 별칭: /api/speedlogs\n  speedlogs: { order: 'ts' }\n};\n\nconst table = p.table;\nif (!ALLOW[table]) {\n  msg.statusCode = 400;\n  msg.payload = { error: \"table not allowed\", allowed: Object.keys(ALLOW) };\n  return msg;\n}\n\n// 정수 sanitize\nconst toInt = (v, d) => { const n = parseInt(v); return Number.isFinite(n) ? n : d; };\nconst limit = Math.min(Math.max(toInt(q.limit, 200), 1), 5000);\nconst offset = Math.max(toInt(q.offset, 0), 0);\n\nif (table === 'speedlogs') {\n  msg.topic = `SELECT id, speed, created_at AS timestamp\\n                 FROM motor_speed1\\n               UNION ALL\\n               SELECT id, speed, created_at AS timestamp\\n                 FROM motor_speed2\\n               ORDER BY timestamp DESC\\n               LIMIT ${limit} OFFSET ${offset}`;\n  delete msg.params;\n  return msg;\n}\n\nconst orderCol = ALLOW[table].order || 'id';\nmsg.topic = `SELECT * FROM ${table} ORDER BY ${orderCol} DESC LIMIT ${limit} OFFSET ${offset}`;\ndelete msg.params;\nreturn msg;",

"outputs": 1,

"timeout": "",

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 520,

"y": 1360,

"wires": [

[

"ac0f25b7c5b4fdcf"

]

]

},

{

"id": "f2d61115cd9349c5",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "Set JSON headers",

"func": "msg.headers = msg.headers || {};\nmsg.headers[\"Content-Type\"] = \"application/json; charset=utf-8\";\nreturn msg;",

"outputs": 1,

"timeout": "",

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 970,

"y": 1360,

"wires": [

[

"2cb44bac8641e442"

]

]

},

{

"id": "2cb44bac8641e442",

"type": "http response",

"z": "5124c2cc8eafaa38",

"name": "",

"statusCode": "",

"headers": {},

"x": 1170,

"y": 1360,

"wires": []

},

{

"id": "ac0f25b7c5b4fdcf",

"type": "mysql",

"z": "5124c2cc8eafaa38",

"mydb": "mysql_config_id",

"name": "Insert to MariaDB",

"x": 770,

"y": 1360,

"wires": [

[

"f2d61115cd9349c5"

]

]

},

{

"id": "3a8f74d34060dd56",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "",

"topic": "motor/speed",

"qos": "2",

"datatype": "auto",

"broker": "af94e945c33ea222",

"nl": false,

"rap": false,

"inputs": 0,

"x": 180,

"y": 180,

"wires": [

[

"ee3a0e445e1678d9"

]

]

},

{

"id": "ee3a0e445e1678d9",

"type": "json",

"z": "5124c2cc8eafaa38",

"name": "속도 JSON 파싱",

"x": 380,

"y": 180,

"wires": [

[

"efa861367314ee64"

]

]

},

{

"id": "efa861367314ee64",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "실시간 속도 저장",

"func": "flow.set(\"lastSpeed\", msg.payload.speed);\nreturn null;",

"outputs": 0,

"timeout": "",

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 620,

"y": 180,

"wires": []

},

{

"id": "warn_mark_center",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "control/warning → 기준시각 저장",

"topic": "control/warning",

"qos": "0",

"datatype": "auto",

"broker": "af94e945c33ea222",

"nl": false,

"rap": true,

"rh": 0,

"inputs": 0,

"x": 240,

"y": 1220,

"wires": [

[

"fn_mark_center_ts"

]

]

},

{

"id": "fn_mark_center_ts",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "flow.lastWarningTs = now()",

"func": "flow.set(\"lastWarningTs\", new Date());\nreturn null;",

"outputs": 0,

"noerr": 0,

"x": 530,

"y": 1220,

"wires": []

},

{

"id": "http_latest_window",

"type": "http in",

"z": "5124c2cc8eafaa38",

"name": "GET /api/window/latest",

"url": "/api/window/latest",

"method": "get",

"upload": false,

"swaggerDoc": "",

"x": 250,

"y": 1280,

"wires": [

[

"fn_build_union_sql"

]

]

},

{

"id": "fn_build_union_sql",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "±1분 단일 SQL(UNION ALL) 생성",

"func": "// 기준시각: 최근 warning(있으면) else 지금\nconst centerJs = flow.get(\"lastWarningTs\") || new Date();\nconst center = new Date(centerJs);\n\n// ±1분\nconst from = new Date(center.getTime() - 60*1000);\nconst to   = new Date(center.getTime() + 60*1000);\n\nfunction toMysqlDT(d){\n  return new Date(d.getTime()-d.getTimezoneOffset()*60000)\n           .toISOString().slice(0,19).replace('T',' ');\n}\nconst s = toMysqlDT(from), e = toMysqlDT(to), c = toMysqlDT(center);\n\n// 하나의 결과셋으로 합치기 (src, t, val1, val2, val3, warning_ts)\nmsg.topic = `\n  SELECT * FROM (\n    SELECT 'cam_brake'       AS src, cb.ts            AS t, CAST(cb.state AS DECIMAL(10,3)) AS val1, NULL AS val2, NULL AS val3, '${c}' AS warning_ts\n      FROM cam_brake cb       WHERE cb.ts BETWEEN '${s}' AND '${e}'\n    UNION ALL\n    SELECT 'cam_accel'       AS src, ca.ts            AS t, CAST(ca.state AS DECIMAL(10,3)) AS val1, NULL, NULL, '${c}'\n      FROM cam_accel ca       WHERE ca.ts BETWEEN '${s}' AND '${e}'\n    UNION ALL\n    SELECT 'roadcell_brake'  AS src, r1.ts            AS t, r1.weight AS val1, NULL, NULL, '${c}'\n      FROM roadcell_readings r1 WHERE r1.ts BETWEEN '${s}' AND '${e}'\n    UNION ALL\n    SELECT 'roadcell_accel'  AS src, r2.ts            AS t, r2.weight AS val1, NULL, NULL, '${c}'\n      FROM roadcell_readings_a r2 WHERE r2.ts BETWEEN '${s}' AND '${e}'\n    UNION ALL\n    SELECT 'accel_brake'     AS src, a1.ts            AS t, a1.ax AS val1, a1.ay AS val2, a1.az AS val3, '${c}'\n      FROM accel_readings a1  WHERE a1.ts BETWEEN '${s}' AND '${e}'\n    UNION ALL\n    SELECT 'accel_accel'     AS src, a2.ts            AS t, a2.ax AS val1, a2.ay AS val2, a2.az AS val3, '${c}'\n      FROM accel_readings_a a2 WHERE a2.ts BETWEEN '${s}' AND '${e}'\n    UNION ALL\n    SELECT 'motor_speed1'    AS src, m1.created_at    AS t, m1.speed AS val1, NULL, NULL, '${c}'\n      FROM motor_speed1 m1    WHERE m1.created_at BETWEEN '${s}' AND '${e}'\n    UNION ALL\n    SELECT 'motor_speed2'    AS src, m2.created_at    AS t, m2.speed AS val1, NULL, NULL, '${c}'\n      FROM motor_speed2 m2    WHERE m2.created_at BETWEEN '${s}' AND '${e}'\n  ) x\n  ORDER BY warning_ts, t, src`;\n\n// 결과를 HTTP로 내보낼 거라 params/headers는 다음 노드에서 처리\ndelete msg.params;\nreturn msg;",

"outputs": 1,

"timeout": "",

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 540,

"y": 1280,

"wires": [

[

"mysql_union_exec"

]

]

},

{

"id": "mysql_union_exec",

"type": "mysql",

"z": "5124c2cc8eafaa38",

"mydb": "mysql_config_id",

"name": "MariaDB (union window)",

"x": 800,

"y": 1280,

"wires": [

[

"fn_json_headers",

"debug_union_preview"

]

]

},

{

"id": "fn_json_headers",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "Set JSON headers",

"func": "msg.headers = msg.headers || {};\nmsg.headers[\"Content-Type\"] = \"application/json; charset=utf-8\";\nreturn msg;",

"outputs": 1,

"noerr": 0,

"x": 1030,

"y": 1250,

"wires": [

[

"http_union_resp"

]

]

},

{

"id": "http_union_resp",

"type": "http response",

"z": "5124c2cc8eafaa38",

"name": "",

"statusCode": "",

"headers": {},

"x": 1230,

"y": 1250,

"wires": []

},

{

"id": "debug_union_preview",

"type": "debug",

"z": "5124c2cc8eafaa38",

"name": "결과 미리보기",

"active": true,

"tosidebar": true,

"console": false,

"tostatus": false,

"complete": "payload",

"statusVal": "",

"statusType": "auto",

"x": 1040,

"y": 1310,

"wires": []

},

{

"id": "warn_mqtt_in",

"type": "mqtt in",

"z": "5124c2cc8eafaa38",

"name": "control/warning (logger)",

"topic": "control/warning",

"qos": "0",

"datatype": "auto",

"broker": "af94e945c33ea222",

"nl": false,

"rap": true,

"rh": 0,

"inputs": 0,

"x": 220,

"y": 1420,

"wires": [

[

"warn_fn_to_sql"

]

]

},

{

"id": "warn_fn_to_sql",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "INSERT INTO warning_events",

"func": "let s = (Buffer.isBuffer(msg.payload) ? msg.payload.toString('utf8') : String(msg.payload||'')).trim();\n// 'warning'일 때만 기록 (필요하면 주석 해제하여 모두 기록 가능)\nif (!/^warning$/i.test(s)) return null;\nmsg.topic = \"INSERT INTO warning_events (payload, ts) VALUES ('warning', NOW(6))\";\nreturn msg;",

"outputs": 1,

"timeout": "",

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 510,

"y": 1420,

"wires": [

[

"warn_mysql_insert",

"warn_debug"

]

]

},

{

"id": "warn_mysql_insert",

"type": "mysql",

"z": "5124c2cc8eafaa38",

"mydb": "mysql_config_id",

"name": "MariaDB (warning_events)",

"x": 800,

"y": 1420,

"wires": [

[]

]

},

{

"id": "warn_debug",

"type": "debug",

"z": "5124c2cc8eafaa38",

"name": "Logged warning → DB",

"active": true,

"tosidebar": true,

"console": false,

"tostatus": true,

"complete": "true",

"targetType": "full",

"statusVal": "",

"statusType": "auto",

"x": 830,

"y": 1460,

"wires": []

},

{

"id": "c3a8d10c5f4836f6",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "엑셀 대시보드 on off 변환",

"func": "// payload가 \"accel : 0\" 또는 \"accel : 1\" 형태로 들어옴\nlet s = msg.payload.toString();\n\n// 정규식으로 숫자(0 또는 1)만 추출\nlet m = s.match(/\\b([01])\\b/);\n\nif (!m) {\n    node.warn(\"invalid accel payload: \" + s);\n    return null; // 잘못된 값이면 무시\n}\n\nlet foot = Number(m[1]);\n\n// 표시 변환\nif (foot === 1) {\n    msg.payload = \"ON\";   // 또는 \"발 올림\"\n} else {\n    msg.payload = \"OFF\";  // 또는 \"발 없음\"\n}\n\nreturn msg;\n",

"outputs": 1,

"timeout": 0,

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 770,

"y": 460,

"wires": [

[

"ac7b8b05731d8de8"

]

]

},

{

"id": "58bd6de13e07c03a",

"type": "function",

"z": "5124c2cc8eafaa38",

"name": "브레이크 대시보드 on off 변환",

"func": "// payload가 \"accel : 0\" 또는 \"accel : 1\" 형태로 들어옴\nlet s = msg.payload.toString();\n\n// 정규식으로 숫자(0 또는 1)만 추출\nlet m = s.match(/\\b([01])\\b/);\n\nif (!m) {\n    node.warn(\"invalid brake payload: \" + s);\n    return null; // 잘못된 값이면 무시\n}\n\nlet foot = Number(m[1]);\n\n// 표시 변환\nif (foot === 1) {\n    msg.payload = \"ON\";   // 또는 \"발 올림\"\n} else {\n    msg.payload = \"OFF\";  // 또는 \"발 없음\"\n}\n\nreturn msg;\n",

"outputs": 1,

"timeout": 0,

"noerr": 0,

"initialize": "",

"finalize": "",

"libs": [],

"x": 780,

"y": 220,

"wires": [

[

"dbde57746b297ebb"

]

]

},

{

"id": "ce0405cddd133f30",

"type": "ui_spacer",

"z": "5124c2cc8eafaa38",

"name": "spacer",

"group": "",

"order": 2,

"width": 1,

"height": 1

},

{

"id": "08f0486f70b9dc3f",

"type": "ui_group",

"name": "SpeedPanel",

"tab": "e45b6f346a3baf8f",

"order": 1,

"disp": true,

"width": "7",

"collapse": false,

"className": ""

},

{

"id": "af94e945c33ea222",

"type": "mqtt-broker",

"name": "",

"broker": "192.168.0.6",

"port": 1883,

"clientid": "",

"autoConnect": true,

"usetls": false,

"protocolVersion": 4,

"keepalive": 60,

"cleansession": true,

"autoUnsubscribe": true,

"birthTopic": "",

"birthQos": "0",

"birthRetain": "false",

"birthPayload": "",

"birthMsg": {},

"closeTopic": "",

"closeQos": "0",

"closeRetain": "false",

"closePayload": "",

"closeMsg": {},

"willTopic": "",

"willQos": "0",

"willRetain": "false",

"willPayload": "",

"willMsg": {},

"userProps": "",

"sessionExpiry": ""

},

{

"id": "mysql_config_id",

"type": "MySQLdatabase",

"name": "MariaDB",

"host": "localhost",

"port": "3306",

"db": "mydb",

"tz": "",

"charset": "UTF8"

},

{

"id": "e45b6f346a3baf8f",

"type": "ui_tab",

"name": "Dashboard",

"icon": "dashboard",

"order": 1

}

]